数据通信NS3作业-2

姓名： 费扬  学号：   519021910917     日期：  2022.05.09

- 实验名称及内容

Lab2：

Build a 2-hop Point-to-Point network as illustrated below,

192.168.10.0          192.168.50.0

n0 ------------------------ n1-------------------------------- n2

point-to-point           point-to-point

5Mbps, 2ms            1Mbps, 2ms

- n0 sends a total number of 2000 bytes to n2.

- Use onoff-application (TCP) on n0 with packet size 512, data rate 50kb/s, set the OnTime random variable to 1 and OffTime random variable to 0.

- Use PacketSink (TCP) application on n2 to receive the packets.

- Enable NS_LOG on both onoff-application and PacketSink, turn on pacp tracing on all nodes.

- Use filename: lab2.cc

Hints: For onoff-application and packet-sink application, the source codes are in /home/workspace/ns-allinone-3.28/ns-3.28/src/applications/model. Or go to https://www.nsnam.org/doxygen/index.html, click ModulesApplicationsOnOffApplication to see a detailed description of the application.

- 实验过程和结果

程序见压缩包内。本次ns3的版本为3.30。

Simulation：

- $find . -name '*.cc' | xargs grep OnOffApplication | grep example

- $./waf --run "scratch/mysecond --PrintAttributes=ns3::OnOffApplication"

- $./waf --run scratch/lab2

- pcap file contents to show that packets are delivered to the destination

- Check in Wireshark:

- 实验思考：

- 关于512分片中出现536和488长度分组的情况：

分析是由于默认ns-3的TCP分段长度为536，猜测是按断续发送512长度，在最后一段发送时整个发送了1024长度的报文，默认分片成536，剩余488分组。